Make the Most of your Good Times

540 South Prospect Ave. * Hartville, Ohio 44632

www.hartvillerv.com

Akron/Canton: 330.877.3500   *   Toll Free: 800.686.0201

WINTERIZING

(Steps 1-4)

- Empty all tanks (fresh water, gray water, sewage, water heater, shower pan and toilet).

- Open low point drains.

- Close low point drains.

- Blow air thru lines using an air hose and blow out plug.

Water Heater Bypass with Non-Toxic Anti-Freeze

( Steps 5-11)

- Turn off pump.

- Close valves A, B, and D.  Open valves E and C. (refer to diagram below)

- Drain the water heater and fresh water tank.

- Put the hose from Valve E into a jug of Non-Toxic RV antifreeze.

(You will need more than one gallon of non-toxic RV antifreeze).

- Turn on the pump and open each faucet until the antifreeze flows through (including out-side shower).  Do not forget to run the antifreeze through the toilet.

- Pour at least one cup of non-toxic RV antifreeze into each drain to protect the P-traps.

- Close Valve E.

WATER HEATER BYPASS

WATER PUMP

LOW POINT DRAINS